Preventive maintenance checklist: Pumps (centrifugal)

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Check the seal, gaskets and pipe joints for leaks | Operator | ☐ | ☐ |  |  |  |
| Listen for unusual noise or cavitation (a gravel sound) and feel for vibration | Operator | ☐ | ☐ |  |  |  |
| Read suction and discharge pressure and compare with the normal reading | Operator | ☐ | ☐ |  |  |  |
| Check bearing housing temperature by touch or with an infrared thermometer | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Check bearing oil level and condition; top up with the oil the manual names | Technician | ☐ | ☐ |  |  |  |
| Check packing leak-off and adjust the gland, on pumps with packing | Technician | ☐ | ☐ |  |  |  |
| Clean the suction strainer if the pressure readings show it is blocking | Technician | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Grease the bearings with the grease type and quantity in the manual | Technician | ☐ | ☐ |  |  |  |
| Check the coupling guard, the coupling and the baseplate bolts | Technician | ☐ | ☐ |  |  |  |
| Take vibration readings at each bearing and log them | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Check pump to motor shaft alignment | Technician | ☐ | ☐ |  |  |  |
| Measure motor current and compare with the nameplate and past readings | Electrician | ☐ | ☐ |  |  |  |
| Check the coupling element for wear and replace it if worn | Technician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Inspect the impeller and wear rings for wear and erosion; check clearances against the manual | Technician | ☐ | ☐ |  |  |  |
| Change the bearing oil, or at the interval the manual gives | Technician | ☐ | ☐ |  |  |  |
| Check pump performance against its curve: flow, head and power | Technician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Air compressors

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Drain condensate from the receiver and filters, or check that the automatic drains work | Operator | ☐ | ☐ |  |  |  |
| Check discharge pressure and temperature against the normal readings | Operator | ☐ | ☐ |  |  |  |
| Check the oil level on oil-lubricated units | Operator | ☐ | ☐ |  |  |  |
| Listen for air leaks and unusual noise | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Check the intake air filter and its pressure drop indicator | Technician | ☐ | ☐ |  |  |  |
| Clean the cooler fins and keep the area around the compressor clear | Technician | ☐ | ☐ |  |  |  |
| Check belt tension and condition on belt-driven units | Technician | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Check that the safety relief valve operates, using the test method in the manual | Technician | ☐ | ☐ |  |  |  |
| Survey the air network for leaks with an ultrasonic detector or soapy water | Technician | ☐ | ☐ |  |  |  |
| Inspect hoses, fittings and condensate drain lines | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Check the oil filter and separator pressure drop; replace elements as the manual specifies | Technician | ☐ | ☐ |  |  |  |
| Take an oil sample for analysis on oil-lubricated units | Technician | ☐ | ☐ |  |  |  |
| Check electrical connections and motor mountings for tightness | Electrician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Change the compressor oil, or at the running hours the manual gives | Technician | ☐ | ☐ |  |  |  |
| Clean the cooler cores and check the thermostatic valve | Technician | ☐ | ☐ |  |  |  |
| Have the air receiver inspected as your local pressure vessel rules require | Specialist | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Conveyors

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Check belt tracking and look for edge damage or tears | Operator | ☐ | ☐ |  |  |  |
| Clear material build-up at pulleys, rollers and transfer points, with the conveyor stopped and locked out | Operator | ☐ | ☐ |  |  |  |
| Listen for noisy rollers or bearings | Operator | ☐ | ☐ |  |  |  |
| Check that all guards are in place | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Test the emergency stops and pull-cord switches | Technician | ☐ | ☐ |  |  |  |
| Check belt tension and the condition of the splice | Technician | ☐ | ☐ |  |  |  |
| Clean photo-eyes and sensors | Operator | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Lubricate bearings and chains as the manual specifies | Technician | ☐ | ☐ |  |  |  |
| Check that rollers turn freely and replace seized ones | Technician | ☐ | ☐ |  |  |  |
| Inspect and adjust belt scrapers and skirt rubbers | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Check the drive gearbox oil level and breather | Technician | ☐ | ☐ |  |  |  |
| Check pulley lagging and pulley alignment | Technician | ☐ | ☐ |  |  |  |
| Check motor current and the drive connections | Electrician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Measure belt wear and plan the replacement | Technician | ☐ | ☐ |  |  |  |
| Inspect the frame, supports and bolts for damage and corrosion | Technician | ☐ | ☐ |  |  |  |
| Take a thermal image of the drive panel and motor connections | Electrician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Electric motors

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Listen for unusual noise and feel for vibration | Operator | ☐ | ☐ |  |  |  |
| Check the motor is not running hotter than usual, by touch or with an infrared thermometer | Operator | ☐ | ☐ |  |  |  |
| Keep the fan cover and cooling fins clear of dust and debris | Operator | ☐ | ☐ |  |  |  |
| Check for a burning smell or discoloration at the terminal box | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Clean the motor frame and fan cover | Operator | ☐ | ☐ |  |  |  |
| Check the mounting bolts for looseness | Technician | ☐ | ☐ |  |  |  |
| Check cables, glands and conduit for damage | Electrician | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Measure running current on each phase and compare with the nameplate | Electrician | ☐ | ☐ |  |  |  |
| Take vibration readings at both bearings and log them | Technician | ☐ | ☐ |  |  |  |
| Check belt tension or coupling alignment to the driven machine | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Grease the bearings with the grease and quantity the manual gives; do not over-grease | Technician | ☐ | ☐ |  |  |  |
| Take a thermal image of the terminal box and the starter or drive connections | Electrician | ☐ | ☐ |  |  |  |
| Check the earth (ground) connection | Electrician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Test insulation resistance and log it so the trend shows | Electrician | ☐ | ☐ |  |  |  |
| Check terminal connections are tight to the torque in the manual | Electrician | ☐ | ☐ |  |  |  |
| Inspect the bearings and plan a replacement if noise or vibration readings are rising | Technician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Gearboxes

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Check for oil leaks at seals and joints | Operator | ☐ | ☐ |  |  |  |
| Listen for unusual noise and feel for heat or vibration | Operator | ☐ | ☐ |  |  |  |
| Check the oil level in the sight glass | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Check that the breather is clean and not blocked | Technician | ☐ | ☐ |  |  |  |
| Clean the housing so it can shed heat | Operator | ☐ | ☐ |  |  |  |
| Check the mounting bolts | Technician | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Measure housing temperature with an infrared thermometer and log it | Technician | ☐ | ☐ |  |  |  |
| Take vibration readings and log them | Technician | ☐ | ☐ |  |  |  |
| Check the couplings and their guards | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Take an oil sample for analysis | Technician | ☐ | ☐ |  |  |  |
| Check the shaft seals and replace any that are weeping | Technician | ☐ | ☐ |  |  |  |
| Check backlash at the output shaft and log it | Technician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Change the oil, or at the interval the manual or the oil analysis gives | Technician | ☐ | ☐ |  |  |  |
| Inspect the gear teeth through the inspection cover for wear and pitting | Technician | ☐ | ☐ |  |  |  |
| Check alignment with the motor and the driven machine | Technician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Filling machines

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Clean and sanitize product contact parts as your cleaning procedure requires | Operator | ☐ | ☐ |  |  |  |
| Check fill weights or volumes against the specification at start-up | Operator | ☐ | ☐ |  |  |  |
| Check the nozzles for drips, blockages and damage | Operator | ☐ | ☐ |  |  |  |
| Check that guards, doors and interlocks work | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Inspect seals, O-rings and gaskets on valves and nozzles | Technician | ☐ | ☐ |  |  |  |
| Lubricate the points in the manual, with food grade lubricant where required | Technician | ☐ | ☐ |  |  |  |
| Clean and check sensors and photo-eyes | Operator | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Verify or calibrate the fill measurement (scale or flow meter) | Technician | ☐ | ☐ |  |  |  |
| Check star wheels, guides and change parts for wear | Technician | ☐ | ☐ |  |  |  |
| Check pneumatic filters, regulators and lines for leaks | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Replace the wear seals on the fill valves as the manual specifies | Technician | ☐ | ☐ |  |  |  |
| Check drive chains, belts and the gearbox | Technician | ☐ | ☐ |  |  |  |
| Check the timing between infeed, filling and capping | Technician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Overhaul the fill valves | Technician | ☐ | ☐ |  |  |  |
| Inspect the turret and drive bearings | Technician | ☐ | ☐ |  |  |  |
| Review the change parts and the critical spares | Technician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Packaging machines

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Clean film, glue and product residue from the forming, sealing and cutting areas | Operator | ☐ | ☐ |  |  |  |
| Check seal quality on the first packs and the sealing jaws for build-up | Operator | ☐ | ☐ |  |  |  |
| Check that guards, doors and interlocks work | Operator | ☐ | ☐ |  |  |  |
| Check the air pressure at the main regulator | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Clean sensors, photo-eyes and the print mark reader | Operator | ☐ | ☐ |  |  |  |
| Check knives and cutters for damage and sharpness | Technician | ☐ | ☐ |  |  |  |
| Lubricate the points in the manual | Technician | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Check belts, chains and timing belts for wear and tension | Technician | ☐ | ☐ |  |  |  |
| Check the sealing temperature against the set point with a calibrated thermometer | Technician | ☐ | ☐ |  |  |  |
| Inspect suction cups and vacuum pads | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Check bearings, cams and cam followers | Technician | ☐ | ☐ |  |  |  |
| Check pneumatic cylinders for leaks and end cushioning | Technician | ☐ | ☐ |  |  |  |
| Check servo drive and motor connections | Electrician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Replace wear parts such as sealing jaw coatings and knives as the manual specifies | Technician | ☐ | ☐ |  |  |  |
| Check the gearbox oil | Technician | ☐ | ☐ |  |  |  |
| Test the whole safety circuit and every interlock | Electrician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: CNC machines

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Check the way lube and spindle lubrication levels | Operator | ☐ | ☐ |  |  |  |
| Check coolant level and concentration with a refractometer | Operator | ☐ | ☐ |  |  |  |
| Clear chips from the work area and check the chip conveyor | Operator | ☐ | ☐ |  |  |  |
| Check the air pressure and drain the air filter bowl | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Clean the coolant tank screens and skim off tramp oil | Operator | ☐ | ☐ |  |  |  |
| Clean the spindle taper and the tool holder tapers | Operator | ☐ | ☐ |  |  |  |
| Check the door window, the door interlock and the work light | Operator | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Clean the electrical cabinet filters and check the cabinet fans | Electrician | ☐ | ☐ |  |  |  |
| Check hydraulic oil level and pressure, where fitted | Technician | ☐ | ☐ |  |  |  |
| Check way covers and wipers for damage | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Check that the machine is level | Technician | ☐ | ☐ |  |  |  |
| Check spindle runout with a test bar and dial indicator | Technician | ☐ | ☐ |  |  |  |
| Measure the backlash on each axis and log it | Technician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Empty and clean the coolant tank and refill with fresh coolant | Technician | ☐ | ☐ |  |  |  |
| Check axis accuracy with a ballbar or laser | Specialist | ☐ | ☐ |  |  |  |
| Change hydraulic oil and filters, and check the backup batteries, as the manual specifies | Technician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Injection molding machines

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Check hydraulic oil level and temperature | Operator | ☐ | ☐ |  |  |  |
| Check mold cooling water flow and temperature | Operator | ☐ | ☐ |  |  |  |
| Check for oil, water and material leaks | Operator | ☐ | ☐ |  |  |  |
| Check that the safety gate and its interlocks work | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Check heater band temperatures against the set points | Technician | ☐ | ☐ |  |  |  |
| Check the automatic lubrication of the toggle and tie bars, or lubricate as the manual specifies | Technician | ☐ | ☐ |  |  |  |
| Clean the machine, the hopper area and the dryer filter | Operator | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Check heater bands and thermocouples; replace any that have failed | Electrician | ☐ | ☐ |  |  |  |
| Clean the oil cooler and check the water filters | Technician | ☐ | ☐ |  |  |  |
| Inspect hydraulic hoses and fittings | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Take a hydraulic oil sample for analysis | Technician | ☐ | ☐ |  |  |  |
| Watch screw recovery time and cushion for signs of screw or check ring wear | Technician | ☐ | ☐ |  |  |  |
| Check electrical cabinet connections and fans | Electrician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Measure screw and barrel wear | Specialist | ☐ | ☐ |  |  |  |
| Check platen parallelism and the condition of the tie bars | Specialist | ☐ | ☐ |  |  |  |
| Replace the hydraulic filters, and the oil if the analysis calls for it | Technician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.

Preventive maintenance checklist: Hydraulic systems

Free template from oeelab.com/preventive-maintenance-checklist-template. Updated October 2026.

| Asset name or ID | Location or line | Week or month | Checklist owner |
| --- | --- | --- | --- |

Before you start: isolate and lock out the machine before any task inside guards or covers. Follow the manufacturer's manual for intervals, lubricants, quantities and torque values.

| Task | Who | Done | Problem | Done by | Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Daily |  |  |  |  |  |  |
| Check the oil level and temperature | Operator | ☐ | ☐ |  |  |  |
| Look for leaks at hoses, fittings and cylinders | Operator | ☐ | ☐ |  |  |  |
| Read the system pressure and compare with the normal reading | Operator | ☐ | ☐ |  |  |  |
| Listen for pump noise that sounds like cavitation | Operator | ☐ | ☐ |  |  |  |
| Weekly |  |  |  |  |  |  |
| Check the filter condition indicators | Technician | ☐ | ☐ |  |  |  |
| Check the reservoir breather | Technician | ☐ | ☐ |  |  |  |
| Check hoses for abrasion, kinks and bulges | Technician | ☐ | ☐ |  |  |  |
| Monthly |  |  |  |  |  |  |
| Clean the oil cooler | Technician | ☐ | ☐ |  |  |  |
| Check the accumulator pre-charge, by a trained person following the manual | Technician | ☐ | ☐ |  |  |  |
| Check cylinder rods for damage and rod seals for leaks | Technician | ☐ | ☐ |  |  |  |
| Quarterly |  |  |  |  |  |  |
| Take an oil sample for analysis (particle count and water) | Technician | ☐ | ☐ |  |  |  |
| Change the filters when the indicator or the manual calls for it | Technician | ☐ | ☐ |  |  |  |
| Check the pump to motor coupling and alignment | Technician | ☐ | ☐ |  |  |  |
| Annually |  |  |  |  |  |  |
| Change the oil if the analysis calls for it | Technician | ☐ | ☐ |  |  |  |
| Inspect and clean the inside of the reservoir | Technician | ☐ | ☐ |  |  |  |
| Replace hoses that are damaged or past the service life the hose maker gives | Technician | ☐ | ☐ |  |  |  |

Mark Problem when a check finds something, and raise a work order for it. The frequencies are a common starting point; your manual and your failure history set the real intervals.